DATE

1 December 2025

REFERENCE 0792129

APPENDIX B: AIR MONITORING ANALYTICAL RESULTS

Bauxite III Sampling Results (As of 11/28/25)

| Agent | Units | Count | Metals |  | PAH (Benzo(a)pyrene) | Hydrogen Cyanide | Fluorides | PCB (Aroclor 1242) | Sample Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Method |  |  | NIOSH 7303 (Inorganic As)NIOSH 7303 (Total Cr)NIOSH 5506 |  |  | NIOSH 7904 | NIOSH 7902 | NIOSH 5503 |  |
| OSHA PEL | mg/m3 |  | >0.01 >0.5 |  | >0.2 | >11 | >2.5 | >0.5 |  |
| 11/13/2025 | mg/m3 | 5 | <0.00063 <0.0047 (A) |  | <0.00095 (A) | <0.022 (A) | <0.0042 (A) | <0.0012 (A) | On 4ft stilt, approximetally 5ft from where Jemco staff installing explosives for blasting, downwind. |
| 11/12/2025 | mg/m3 | 5 | <0.00065 <0.0049 (A) |  | <0.00099 (A) | <0.023 (A) | <0.0043 (A) | <0.0012 (A) | South-central of Pad A during installation of DelMarva's SE sanitary line, area sample on perimeter |
| 11/10/2025 | mg/m3 | 5 | <0.00063 <0.0047 (P) |  | <0.00095 (P) | <0.022 (P) | <0.0042 (P) | <0.0012 (P) | Personal samples on DelMarva trench worker Christian (Metals), trench worker Juan Fuentes (PCB),excavator operator Luis Viera (F), trench worker Alfredo (PAH), trench worker Robin Chavez (HC) |
| 11/7/2025 | mg/m3 | 5 | <0.00065 <0.0049 (A) |  | <0.00099 (A) | <0.023 (A) | <0.0043 (A) | <0.0012 (A) | S Central of Pad A, DelMarva Excavation Area, N of Work Area |
| 11/5/2025 | mg/m3 | 5 | <0.00065 <0.0049 (A) |  | <0.00099 (A) | <0.023 (A) | <0.0043 (A) | <0.0012 (A) | Area samples S of Pad A, E Side of DelMarva Excavation Area |
| 11/4/2025 | mg/m3 | 5 | <0.00067 <0.0050 (A) |  | <0.0010 (A) | <0.021 (A) | <0.0044 (A) | <0.0012 (A) | Area samples S of Pad A, NE Side of DelMarva Excavation Area |
| 10/28/2025 | mg/m3 | 5 | <0.00069 <0.0052 (P) |  | <0.00082 (P) | <0.021 (P) | <0.0042 (P) | <0.0012 (P) | Personal samples on trench workers Carlos Hernandez (M), Juan Jose Fuentes (PCB); Excavator operators RobertNavarrette (HC), Oscar Dominguez (F); and supply support Jesus Jimenez (PAH) |
| 10/27/2025 | mg/m3 | 5 | <0.00063 <0.0047 (A) |  | <0.00095 (A) | <0.022 (A) | <0.0042 (A) | <0.0012 (A) | Area sample N of Delmarva excavation located on S side of field site. |
| 10/23/2025 | mg/m3 | 5 | <0.00063 <0.0047 (P) |  | <0.00095 (P) | <0.022 (P) | <0.0042 (P) | <0.0012 (P) | Five personal samples on five Delmarva staff during trenching in the SW corner of the field site during installation ofstormwater drain. |
| 10/22/2025 | mg/m3 | 5 | <0.00065 <0.0049 (A) |  | <0.00098 (A) | <0.023 (A) | <0.0043 (A) | <0.0012 (A) | Five area samples co-located, SW corner of field site, adjacent to and within 1 ft S of the Delmarva excavation forinstallation of stormwater pipe. Co-located |
| 10/21/2025 | mg/m3 | 5 | <0.00067 <0.0050 (A) |  | <0.0010 (A) | <0.023 (A) | <0.0045 (A) | <0.0013 (A) | Five area samples co-located, SW corner of field site, adjacent to and within 1 ft S of the Delmarva excavation forinstallation of stormwater pipe |
| 10/20/2025 | mg/m3 | 5 | <0.00063 <0.0047 (A) |  | <0.00095 (A) | <0.022 (A) | <0.0042 (A) | <0.0012 (A) | Five area samples co-located, mounted inside Keller excavator operated by Brian Thomas during drilling at Pad A.Keller moved across whole pad, east to west, during monitoring |
| 10/17/2025 | mg/m3 | 5 | <0.00068 <0.0051 (A) |  | <0.0010 (A) | <0.024 (A) | <0.0045 (A) | <0.0013 (A) | Five area samples co-located within 10 ft downwind of Keller drilling operation on pad A. Keller went across wholepad from east to west during monitoring |
| 10/16/2025 | mg/m3 | 4 | NA | NA | <0.00095 (A) | <0.022 (A) | <0.0042 (P) | <0.0012 (P) | PAH, and HC area samples co-located and mounted on burm directly north of Delmarva staging area. PCB and F arepersonal samples mounted during survey work on pad A. Metals sample cassette had missing MCE filter. |
| 10/15/2025 | mg/m3 | 4 | <0.00063 <0.0047 (A) |  | <0.00095 (A) | <0.022 (A) | <0.0042 (A) | NA | Four co-located area samples mounted on the pedestrian Walkway near turn onto eastern access road before SMAboundary, just north of Keller trailers. |
| 10/9/2025 | mg/m3 | 4 | <0.00063 <0.0047 (P) |  | <0.00095 (A) | NA | <0.0042 (A) | <0.0012 (P) | Metal and PCB are personal samples mounted on Keller drill operators, PAH and F are area samples mounted on aircompressor about 30 ft from drill point. Keller operating on east side of pad C |
| 10/8/2025 | mg/m3 | 5 | <0.00063 <0.0047 (x2A) |  | <0.00095 (x2A) | <0.022 (A) | NA | NA | Two metal area samples, two PAH area samples, and an HC area sample mounted on pressure wash engine about10ft from SMA pressure wash operation. |
| 10/7/2025 | mg/m3 | 5 | <0.00063 <0.0047 (P+A) |  | <0.00095 (x2A) | <0.022 (A) | NA | NA | Personal metal sample mounted on the SMA Turner wash station operator, four area samples, a metal, two PAHs,and an HC mounted on wash station canopy within 10ft of wash station operation. |
| 10/1/2025 | mg/m3 | 5 | <0.00063 <0.0047 (A) |  | <0.00095 (A) | <0.022 (A) | NA | <0.0012 (x2A) | Five area samples mounted on outside railing of Pleasants bullbozer operated by Matt during grading of pads |
| 9/30/2025 | mg/m3 | 4 | <0.00063 <0.0047 (A) |  | <0.00095 (A) | <0.022 (A) | NA | <0.0012 (A) | Four area samples mounted on outside railing of Pleasants bullbozer during grading of pads A and C. A duplicatePCB filter was hung on the bulldozer, but this sample train fell during grading. |
| 9/29/2025 | mg/m3 | 5 | <0.00063 <0.0047 (A) |  | <0.00095 (A) | <0.022 (A) | NA | <0.0012 (x2A) | Five area samples installed on E side of trailer city excavation between admin zone & excavation. Installed on trailerwater tanks |
| 9/24/2025 | mg/m3 | 5 | <0.00071 <0.0054 (A) |  | <0.0011 (x2A) | <0.025 (A) | NA | <0.0013 (A) | Five area samples mounted on posts in the center of the field site at height of 4ft during Pleasants excavation andgrading of pads A and C. |
| 9/23/2025 | mg/m3 | 5 | <0.00083 <0.0063 (P) |  | <0.0013 (P) | <0.029 (A) | <0.0056 (P) | <0.0016 (P) | Four personal samples (M, PCB, F, PAH) mounted on Hercules staff during fencing installation on the north side ofthe field site. One HC area sample mounted on the fence within 5ft of work |

Page 1 of 2

| Agent | Units | Count | Metals |  | PAH (Benzo(a)pyrene) | Hydrogen Cyanide | Fluorides | PCB (Aroclor 1242) | Sample Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9/22/2025 | mg/m3 | 5 | <0.00079 <0.0059 (P) |  | NA | NA | NA | <0.0015 (x4P) | Five personal samples mounted on Hercules staff in NW corner of field site during perimeter fence installation. Workperformed adjacent to Pleasants leveling W soil piles. |
| 9/18/2025 | mg/m3 | 5 | <0.00083 <0.0063 (A) |  | <0.0013 (A) | <0.029 (A) | <0.0056 (A) | <0.0016 (P) | Four area samples mounted on SMA wash station canopy, one personal PCB sample mounted on Turner wash stationoperator. |
| 9/16/2025 | mg/m3 | 5 | <0.00083 <0.0063 (A) |  | <0.0013 (A) | <0.029 (A) | <0.0056 (A) | <0.0016 (P) | Four area samples 10 ft downwind of Hawkins installation of snow fence, one personal PCB sample on Hawkins staff.Work takes place E of SMA wash station. |
| 9/15/2025 | mg/m3 | 4 | <0.00050 <0.0038 (A) |  | <0.00076 (A) | NA | <0.0033 (A) | <0.00094 (A) | Five area samples W side of field site by Hercules fence installers, on 4ft stakes, N side of W soil piles. Hydrogencyanide sample liquid vial broke during shipment |
| 9/11/2025 | mg/m3 | 5 | <0.00063 <0.0047 (A) |  | <0.00095 (A) | <0.022 (A) | <0.0042 (A) | <0.0012 (A) | Five area samples mounted in Keller Excavator Cab driven by Rob during work on pads A and C. |
| 9/10/2025 | mg/m3 | 5 | <0.00063 <0.0047 (A) |  | <0.00095 (A) | <0.022 (A) | <0.0042 (A) | <0.0012 (A) | Five area samples mounted in Keller Excavator Cab driven by David during work on pads A and C |
| 9/9/2025 | mg/m3 | 5 | <0.00054 <0.0040 (A) |  | <0.00081 (A) | <0.019 (A) | <0.0036 (A) | <0.0010 (A) | Five area samples mounted in Keller mini bulldozer driven by David during work on pads A and C |
| 9/5/2025 | mg/m3 | 5 | <0.00073 <0.0055 (A) |  | <0.0011 (A) | <0.025 (A) | <0.0049 (A) | <0.0014 (A) | Five area samples mounted 10 ft downwind of Keller air tracking operation on stake at 4ft. Work took plance in theSW corner of the field site. |
| 9/4/2025 | mg/m3 | 5 | <0.00071 <0.0054 (A) |  | <0.0011 (A) | <0.025 (A) | <0.0048 (A) | <0.00054 (A) | Five area samples mounted on the outside railing of dump truck operated by pleasants Representative Keith.Operation included excavator filling, driving, and dumping soil load. Truck driving across field site east to west and west to east. |
| 9/3/2025 | mg/m3 | 5 | <0.00083 <0.0063 (A) |  | <0.0013 (A) | <0.029 (A) | <0.0056 (A) | <0.0016 (A) | Five area samples mounted on 4ft stakes downwind of excavation of traps 3A and 3B in the S side of the field site. |
| 8/25/2002 | mg/m3 | 3 | <0.00059 <0.0044 (A) |  | <0.00089 (A) | NA | NA | <0.0011 (A) | Three area samples in the SW corner of field site, down wind of excavation of traps 1 & 2 |
| 8/22/2025 | mg/m3 | 2 | <0.00059 <0.0044 (A) |  | NA | NA | NA | <0.0011 (A) | Two area samples mounted in Pleasants excavator cab during excavation of traps 1 and 2 |
| 8/20/2025 | mg/m3 | 4 | <0.00075 <0.0056 (A) |  | <0.0011 (A) | NA | <0.0050 (A) | <0.0014 (A) | Four area samples mounted in the Pleasants excavator cab during excavation of traps 1 and 2 |
| 8/13/2025 | mg/m3 | 2 | <0.00033 <0.0017 (A) |  | NA | NA | NA | <0.00083 (A) | Two area samples mounted in the Pleasants excavator cab during excavation of traps 1 and 2 |
| 8/12/2025 | mg/m3 | 6 | <0.00033 <0.0017 (x4A) |  | NA | NA | NA | <0.00083 (A), <0.00071 (A) | One PCB area sample and three metals area samples mounted on a stake downwind of excavation of traps 1 and 2,One PCB and one metal samples mounted inside Pleasants excavator cab during traps 1 and 2 excavation. |
| 8/11/2025 | mg/m3 | 6 | <0.00027 <0.0013 (x4A) |  | NA | NA | NA | <0.00083 (x2A) | One PCB area sample and three metals area samples mounted on a stake downwind of excavation of traps 1 and 2,One PCB and one metal samples mounted inside Pleasants excavator cab during traps 1 and 2 excavation. |
| Sample Count |  |  | 29 22 19 3538 |  |  |  |  |  |  |
| Maximum Measured Value |  |  | <0.00083 | <0.0063 | <0.0013 | <0.029 | <0.0056 | <0.0016 |  |
| PEL / Max |  |  | 12x | 79x | 154x | 379x | 446x | 313x |  |

(A) = Area sample, (P) = Personal Sample, (x#) = multiple samples

Below Limit of Detection

Page 2 of 2