Kane and Lombard Street (NPL Site)

What You Need to Know

# Site Location

The Kane and Lombard site is in Baltimore City at the intersection of Kane and Lombard Streets. The site consists of approximately 9.7 acres south of Lombard Street (Operable Unit 1 – OU1) and approximately 40 acres north of Lombard Street (Operable Unit 2 – OU2). The southern portion (OU1), which is adjacent to Patterson High School, is currently a golf driving range and parking lot. The northern portion (OU2) is used by several commercial properties, including PICORP Inc., which stores shipping containers.

# Site History

Between 1962 and 1964, excavation and dumping occurred on the OU2 portion, north of what later became Lombard Street. These disposal practices proceeded further south onto the OU1 portion between 1964 and 1966. By 1971, much of the disturbed area had been filled and construction of Interstate 95 was underway. Between 1971 and 1982, the excavated areas west, north, and east of the southern portion had been filled and developed.

# Environmental Investigations and Actions

The main contaminants of concern, in both soil and groundwater, are volatile organic compounds (VOCs).

Operable Unit 1 (OU1)

In 1980, hundreds of stacked drums were observed on OU1. After several unsuccessful attempts to compel action by the site owners, Maryland requested assistance from the U.S. Environmental Protection Agency (EPA). In 1984, EPA conducted a Removal Action at OU1, which included the off-site disposal of contaminated surface soil and over 1,000 drums.  Approximately 800 drums were classified as empty, while the contents of the other drums included chlorinated organic compounds, polycyclic aromatic hydrocarbons, phthalates, polychlorinated biphenyl compounds (PCBs), and metals. Following the removal action, a compacted clay layer was installed in the base of the excavation (above the remaining waste fill).

The site was included on the EPA’s National Priorities List (NPL) in June 1986. To facilitate the investigation and cleanup process, EPA designated the operable units described above.

EPA issued the Record of Decision (ROD) for OU1 in 1987, which specified the cleanup action. The selected remedy was a multi-layer cap and slurry wall around the site, combined with dewatering of the first water-bearing zone contained within the wall. Construction of the remedy was completed in August of 1990. Operations and maintenance activities are ongoing for OU1. Any water withdrawn from the OU1 extraction wells is discharged to the Baltimore City sanitary sewer system following pre-treatment.

Operable Unit 2 (OU2)

Investigation of OU2 began in 1985 with a preliminary assessment of the property by Maryland. In 1990, the Maryland Department of the Environment (MDE) initiated a Remedial Investigation (RI) at OU2. This study was discontinued in 1993, when EPA and several potentially responsible parties (PRPs) associated with the previous disposal practices at the site entered into an Administrative Order to perform a Remedial Investigation/Feasibility Study (RI/FS). The initial OU2 RI/FS was completed in July 2002.

In addition to the landfilled portion, OU2 includes a large plume (over one mile long) of chlorinated VOC–contaminated groundwater that extends to the south from OU2.  In 2016, a pilot study for groundwater began operation in the Middle Plume Area.  The pump-and-treat system uses ozone to treat the VOCs and a flocculation/sludge dewatering process to reduce inorganic content in the treated water. The system, which remains in operation as an interim measure, discharges treated groundwater to a local surface water body under a Maryland National Pollutant Discharge Elimination System (NPDES) permit.

In August 2024, EPA issued a Proposed Plan for Remedial Action at OU2, which addresses two small areas on the 40-acre property and the extensive plume of contaminated groundwater.  In September 2025, the ROD for this action was issued by EPA.

[Note: EPA issued an initial OU2 ROD in 2003.  The ROD included a Soil Management Plan (SMP) to address portions of the OU2 landfilled area.  This SMP remains in effect and is being upgraded to comply with the 2025 OU2 ROD.  The 2003 ROD also included a groundwater treatment system (Enhanced Reductive Dechlorination, ERD), which was determined to be unworkable at the site.  Accordingly, EPA initiated a Focused Feasibility Study (FFS) to determine a practicable groundwater remedy.  The FFS was completed in 2022 and the resulting OU2 ROD issued in 2025.]

EPA asked the PRPs to evaluate the potential for vapor intrusion (VI) into buildings on the northern portion of the site. This evaluation resulted in the installation of vapor intrusion (VI) mitigation systems at two of the properties on OU2 in 2016.

# Current Status

On August 31, 2025, a fire occurred on the OU1 site, which the Baltimore Fire Department extinguished.  A follow-up inspection was conducted on September 3, 2025, which ascertained that two of the three main buildings had fire damage. Some debris was observed on the cap, but there were no apparent signs of visible damage.  Further assessment of the potential impact of the fire on the OU1 remedy is being evaluated.

# Planned or Potential Future Action

# The next phase of the cleanup process will be Remedial Design and Remedial Action.