- Overview

This explanatory document is accompanied with the data folder “A3. SRB_load_estimates_output”, which contains concentration and loading estimates at the seven monitoring sites in the Susquehanna River Basin (SRB; Figure 1). These estimates have been extracted from the .Rdata files contained in the data folder “A2. SRB_WRTDS_model_run/R runs”, which were obtained using the WRTDS method (Hirsch et al., 2010). The constituents covered include total nitrogen (TN), dissolved nitrogen (DN), total phosphorus (TP), dissolved phosphorus (DP), and suspended sediment (SS).

Figure 1. Monitoring sites for the sub-basins of the Susquehanna River. This figure was reproduced from Figure 6 in Sprague et al. (2000) with permission, with the original numbering of the monitoring sites. See Table 1 for site details.

- Monitoring Stations and Abbreviations

Characteristics and abbreviations of the monitoring stations are summarized in Table 1. Among the stations, the Conowingo site has been managed by the U.S Geological Survey (USGS) River Input Monitoring (RIM) Program, while the other sites have been managed by the Susquehanna River Basin Commission (SRBC) Sediment and Nutrient Assessment Program (SNAP).

| Table 1. Details of the monitoring stations, adapted from Table 8 in Sprague et al. (2000). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| USGS station number | River and station name | Abbreviation | Drainage area (km2) | Managed by | # on Fig.1 |
| 01531500 | Susquehanna River at Towanda, PA | TOW | 20,194 | SRBC | #2 |
| 01540500 | Susquehanna River at Danville, PA | DAN | 29,008 | SRBC | #3 |
| 01553500 | West Branch Susquehanna River at Lewisburg, PA | LEW | 17,765 | SRBC | #4 |
| 01567000 | Juniata River at Newport, PA | NEW | 8,687 | SRBC | #5 |
| 01576000 | Susquehanna River at Marietta, PA | MAR | 67,314 | SRBC | #7 |
| 01576754 | Conestoga River at Conestoga, PA | CONE | 1,217 | SRBC | #8 |
| 01578310 | Susquehanna River near Conowingo, MD | CONO | 70,189 | USGS | #9 |

- File Details

In total, this data folder contains 245 spreadsheet files, summarizing concentration and loading estimates for FIVE water-quality constituents at the SEVEN monitoring stations at SEVEN different temporal resolutions. The FIVE constituents are total nitrogen (TN), dissolved nitrogen (DN), total phosphorus (TP), dissolved phosphorus (DP), and suspended sediment (SS). The SEVEN temporal resolutions are daily average, monthly average, Jan-Mar seasonal average, Apr-Jun seasonal average, Jul-Sep seasonal average, Oct-Dec seasonal average, and Jan-Dec average (i.e., annual average). All the spreadsheet files are named in the following standard format:

“river abbreviation”_“parameter abbreviation”_“temporal resolution”_“.csv”.

For example, “MAR_DN_Jan-Mar_estimates.csv” contains the WRTDS estimates for river “MAR” (Susquehanna River at Marietta, PA) for water-quality constituent “DN” (dissolved nitrogen) at the temporal resolution “Jan-Mar” (Jan-Mar averages).

In each spreadsheet file, there are six columns, corresponding to time (in years), measured flow discharges Q (in m3/s), true-condition concentration estimates (in mg/L), true-condition loading estimates (in kg/day), flow-normalized concentration estimates (in mg/L), and flow-normalized loading estimates (in kg/day), respectively. Results documented in these spreadsheet files correspond to the estimation periods from the beginning of monitoring (1980s) to early 2013.

- References

Hirsch, R. M., D. L. Moyer, and S. A. Archfield, 2010. Weighted regressions on time, discharge, and season (WRTDS), with an application to Chesapeake Bay river inputs, J. Am. Water Resour. Assoc., 46(5), 857-880, doi: 10.1111/j.1752-1688.2010.00482.x.

Sprague, L. A., M. J. Langland, S. E. Yochum, R. E. Edwards, J. D. Blomquist, S. W. Phillips, G. W. Shenk, and S. D. Preston, 2000. Factors affecting nutrient trends in major rivers of the Chesapeake Bay Watershed, Water-Resources Investigations Report 00-4218, 109 pp, U.S. Geological Survey, Richmond, VA.